PERSONAL PROTECTIVE EQUIPMENT

- Hi-vis hardhat

- Hearing protection

- Gloves

- Suitable footwear

- Hi Vis Vest

- Eye Protection

- Leg Protection

SAFE PROCEDURES

- Bucker is to control traffic on landings.

- Ensure other workers are clear of your hazardous area.

- Watch for pivot points and scissor logs while bucking.

- Stand on the upper side of log situated on an incline.

- Be certain you are not standing on the whip side.

- Logs must not be partially bucked.

- When limbing, stand with both legs on the opposite side of the log.

- Avoid stepping on chunks.

- Ensure danger trees are removed by qualified worker from around the landing.

- Keep clear of loader and skidding machines.

- Never turn your back to approaching equipment. Watch for trees that may hang-up and pivot while being skidded onto the landing.

- Never walk under a log that is being held up by the loader.

- Choose a maintenance area away from other work activity. Go to this safe zone when bucking is completed.

- If all chokers cannot be unhooked, stand well in the clear when the skidder moves ahead to clear the drag.

- Do not start limbing or bucking until chokers have been removed.

- Make sure logs are stable before bucking.