OPERATIONAL SIGNALS FOR HIGH LEAD LOGGING

Ahead
Three short whistles.

Ahead on Strawline *
Three short - pause - one short whistle.

Back **
Two short - pause - two short whistles.

Slack Haulback
Two short - pause - series of short whistles.

Slack Mainline
Series of short whistles.

Slow
One long whistle (precedes any signal for slow operation).

Stop All Lines
One short whistle.

Tighten Lines
Three short - pause - two short whistles.

When Butt Rigging At Tree Send Out Strawline
Three short - one long whistle.

When Butt Rigging Is At Tree
Two short - followed by a number of long whistles indicates the number of chokers required.

- *"Ahead" means haulage line moves toward machine.

- **"Back" means haulage line moves away from machine.

| AUDIBLE HIGH-LEAD SIGNALS OPERATIONAL SIGNALS |  |  |
| --- | --- | --- |
| Start work | 1 long | ____ |
| Stop any movement | 1 short | . |
| Ahead on mainline | 3 short | … |
| Slack the mainline | 5 short | ….. |
| Ahead on the haulback | 2 short, 2 short | .. .. |
| Slack the haulback | 2 short, several short | .. …. |
| Tightline | 3 short, 2 short | … .. |
| Tightline on inhaul | 3 short, 2 short | … .. |
| Cancel tightline on inhaul | 3 short | … |
| Ahead on strawline | 3 short, 1 short | … . |
| Slack the strawline | 3 short, 1 short, several short | … . …. |
| Pick up the guyline | 2 short, 2 short, 2 short, 1 short | .. .. .. . |
| Slack the guyline | 2 short, 2 short, 2 short | .. .. .. |
| Extreme hazard present | 1 long, sustained until (run-away log, etc.) hazard has stopped or cleared | ___________ |
| Accident | 7 long | __ __ __ __ __ __ __ |
| Fire | 1 long, several short, repeat | ___ . . . . |

| WHEN BUTT RIGGING IS AT THE LANDING |  |  |
| --- | --- | --- |
| Check the rigging | 5 short | ….. |
| Send out strawline extension | 3 short, 1 short, and 1 short for each extension | … .. |
| Send out strawline in the haulback eye | 3 short, 1 long | ... ___ |
| Chokers required | 2 short and 1 short or long for each choker required | .. … |
| Put on/take off scab block | 1 long | ___ |
| Calling foreman | 4 long | ___ ___ ___ ___ |
| Calling hooktender | 3 long | ___ ___ ___ |
| Calling hooktender ;and crew | 3 long, several short | ___ ___ ___ …. |
| Calling for water bag | 1 short, 1 long | . ___ |
| Calling for block and strap | 1 long, 1 short | ___ . |

Any regular signal preceded by a long signal is a “slow” signal

Any signal that the engineer is not sure of is a “stop” signal

AUDIBLE SLACKLINE SIGNALS

Refer to the standard high-lead whistle signals for most line control signals. The following are additional whistle signals to be used for SLACKLINE operations.

| OPERATIONAL SIGNALS |  |  |
| --- | --- | --- |
| Stop outhaul and slack skyline | 1 short | . |
| Pick up the skyline | 1 short, 2 short | . .. |
| Slack the skyline | 5 short | ….. |
| Pick up skyline on inhaul to clear obstruction | 2 short | .. |
| Pick up skidding line after obstruction is cleared | 3 short | … |
| Slack the skidding line | 3 short, several short | … …. |
| CARRIAGE ON OUTHAUL |  |  |
| Hold Skidding line tight, keep coming back until stop signal is given | 3 short | … |
| Hold skidding line tight, slack skyline, keep coming | 2 short | .. |
| Slack skyline faster | 2 short | .. |

“Slack skidding line” signal given as “skyline is slacked” means “slack both lines at the same time”

| WHEN CARRIAGE IS AT HEAD SPAR |  |  |
| --- | --- | --- |
| Send strawline out in choker bell for a dead line | 3 short, 1 short, 2 short, 2 short | … . .. .. |
| Send out that many coils | 3 short, 1 short, (1 short for each coil needed) | … . … |
| Calling second rigger | 2 long, 1 short | ___ ___ . |

SKYLINE CARRIAGE SIGNALS

All standard high-lead and slackline whistle signals apply to carriages.

GRAVITY/SHOTGUN CARRIAGE

Standard slackline whistle signals will apply.

| DROPLINE/ACCUMULATOR CARRIAGE |  |  |
| --- | --- | --- |
| Ahead on carriage skidding line | 3 short | … |
| Slack the carriage skidding line | 3 short, several short | … …… |

RADIO-CONTROLLED MOTOR-DRIVEN SLACK PULLER, SKYLINE LOCK

- These carriages are fitted with and controlled by an onboard computerized radio control system.

- This radio system is operated independently through a transmitter separate from that of the yarder.

- The yarding and carriage frequencies must be separate, registered, and coordinated through the WCB co-ordination system to ensure that one does not interfere with the other or with another operation. Contact the WCB Engineering Department for more information.

- An audible signal must be sounded at the carriage and not at the yarder. This signal must have a tone different from that of the yarder signal.

| RADIO-CONTROLLED MOTOR-DRIVEN SLACK PULLER, SKYLINE LOCK |  |  |
| --- | --- | --- |
| Lock/unlock skyline clamp | 2 short | .. |
| Slack the dropline | 5 short | ….. |
| Stop dropline | 1 short | . |
| Ahead on the carriage skidding line | 3 short | … |
| If fitted with engine controls: |  |  |
| Stop engine | 1 short, 1 long | . ___ |
| Start engine | 1 long, 1 short | ___ . |

Carriages with variable dropline speeds must have a special signal for the speed changes. These signals must be different from standard yarding signals.

| RADIO CONTROLLED MOTORIZED SELF-CONTAINED YARDING CARRIAGE This system is similar to the “radio-controlled motor-driven slack puller, skyline lock” carriage, but does not have a skyline lock. |  |  |
| --- | --- | --- |
| Slack the dropline | 3 short, several short | … …. |
| Stop the dropline | 1 short | . |
| Ahead on dropline | 3 short | … |
| If fitted with engine controls: |  |  |
| Stop engine | 1 short, 1 long | . ___ |
| Start engine | 2 short | .. |

Any signal preceded by a long signal is a “slow” signal.

| MECHANICAL SLACK PULLER |  |  |
| --- | --- | --- |
| Ahead on slack puller | 1 long, 1 short | ___ . |
| Ahead on drop line | 2 short | .. |

When the haulback is used as a running skyline, standard high-lead signals apply.