PERSONAL PROTECTIVE EQUIPMENT

- Protective Leg Wear

- Hearing Protection

- Eye Protection

- Hi-vis Vest

- Hi-vis Hardhat

- Gloves

- Caulked Boots

SAFE PROCEDURES

- Make sure the Yarder and Loader Operator know where you are working at all times.

- Never work under the Mainline and haulback when the turn is being yarded.

- Stay clear of the machine counterweight at all times.

- File tangs must be bent over or a handle put on.

- Always keep a firm grip on the saw.

- Caution must be used to avoid kickbacks, never stand directly behind the saw. Always make sure the end of the bar does not strike an object causing a kickback. Whenever possible, avoid boring into a log.

- Before starting the cut, assess the situation and decide what the log is likely to do when the cut is completed.

- If the log is in an unsafe position to buck, leave it or get a machine to move it to a safer position.

- Watch for side bind or spring in long logs or windfalls.

- Broken ends not to be bucked on load.

- Never stand behind a bucked root, it could fall back.

- Watch for limbs with spring in them; they could strike you when cut off. Don't cut off limbs that may allow logs to roll on you.

- Don't use power saw gas for lighting fires.

- Avoid using a saw above shoulder height unless it's necessary; then caution must be used.

- Maintain the saw in good working order for safety and performance.

- Use a designated safe area for saw maintenance, etc.