PERSONAL PROTECTIVE EQUIPMENT

- Hi-vis Hardhat

- Hi-vis Vest

- Hearing Protection

- Eye Protection

- Leg Protection

- Caulked Boots

Responsibility to Others

- Is responsible for the safety of himself and all others in or around the landing.

- Is responsible for directing traffic into or through the landing.

- Is responsible for directing the landing of logs when necessary.

Chasing

- Keep in the clear of the turn when it is being landed (safe zone).

- Be sure logs are stable before going in to unhook them.

- Unhook bottom logs first and from the upper side when applicable.

- Never stand under the yarding lines when a turn is being yarded.

- Inspect the butt rigging regularly and report badly worn parts.

- Replace badly kinked and jaggered chokers.

- Ensure the yarder operator and loaderman know where you are at all times.

- Do not use gas to light fires.

- Keep landing area neat and tidy and equipment in good condition.

General

- Do not look directly at the line when cutting.

- Use a soft hammer when cutting the line.

- Use eye protection when cutting the line.

- Use caution when using sharp tools.

- Wear leg pads whenever using a power saw.

- Adequate training is required by a qualified person before using a power saw.

- Check chokers for jaggers.