PERSONAL PROTECTIVE EQUIPMENT

- Caulk Boots

- Hi-vis Hardhat

- Gloves

- Stagged Pants

Responsibility to Others

- A Rigging Slinger is directly responsible for the safety of the men he directs.

- Ensure your crew are in a safe position at all times.

- Observe and correct the unsafe work practices of your crew.

- Communicate; tell them what you plan for the next turn.

- Insist that your crew pay attention to the turn while it's on its way in; that way they will be alert for sliding logs, rolling rocks, flying chunks or other hazards.

- Never permit the crew to work in the bight or near tailhold stumps.

- Ensure that the crew is above, behind and well in the clear of every turn before giving a "go ahead" whistle.

Responsibilities in General

- A Rigging Slinger is directly under the supervision of the Hooktender.

- Plan your turns carefully, consider the route you will take and the logs to be choked.

- Inspect the rigging and lines for loose shackles, cracks, worn or damaged eyes, burnt lines or scab straps. Inspect the chokers for jaggers.

- Listen to the whistle, blow a stop immediately and correct if the wrong whistle has been given.

- Be careful not to slack lines too far down without instructing loaders to watch for lines.

- Do not set a turn under the pile if loading is in operation.

- If rigging is hung up on a limb or sapling, clear it by slacking the rigging down before setting the turn.

- Adequate training is required by a qualified person before using a power saw.

- Ensure equipment operators are aware of your intentions.

- Ensure equipment operators are aware of your intentions before entering a landing.

- Never allow yourself or your crew to walk into a landing behind or over a pile while a loader is in the landing.

- Use eye protection when cutting cable.

- Slack off lines before going into hang-up.

- Do not enter turn until butt rigging has been slacked to stop flying chokers.