PERSONAL PROTECTIVE EQUIPMENT

- Caulk Boots

- Hi-vis Hardhat

- Hi-vis Vest

- Gloves

- Hearing Protection as required

Review the regular spar Hooktender's Job Safety Breakdown and WCB Grapple Yarder Handbook.

Back Spar - Cat - Excavator

- When threading the guylines through the tower, watch your hands in pinch points.

- Watch for snags along the timber edge.

- Do not sit on the mobile back spar while yarding is in progress.

- Do not get too much bight on the mobile back spar.

- Ensure that the mobile back spar is in proper position and secured before starting to yard.

- Ensure boom is in lead and blade or bucket securely on ground.

- Watch that the running lines do not dislodge trees, logs or other materials when moving the mobile back spar on steeper logging sites. The back spar operator must be in a safe location before the running lines are tightlined.

- Always enter and leave machine in safe manner. Use the handholds provided.

Communications:

- Use the approved voice terminology and hand signals.

- Radios must be in good working condition.

- When talking operator onto logs, ensure you are well clear of turn - to side and back.

- Make sure you are well clear of lines – remember these machines swing.

- Wear hi-vis vest/hi-vis hard hat.

Rigging

- Check the rigging periodically for loose shackles in the grapple and carriage.

Rigging Backspar Trees:

- When using back spars ensure that proper rigging practices are used.