PERSONAL PROTECTIVE EQUIPMENT

- Hearing Protection

- Hi-vis Hardhat

- Hi-vis Vest when off machine

- Adequate Footwear

Responsibility to Others

- The Yarding Engineer's primary responsibility is to be sure that his actions do not create a hazard to the others working around him or to himself.

Checking Machine:

- Make sure machine is in a safe operating condition.

- Check the whistles to ensure that they are working properly.

- Be sure of your footing while moving around the machine.

- After using any guyline winch or when taking over a new machine, make a visual check of the guyline dogs.

- Always enter and leave machine in safe manner. Use handholds for stability.

Signals:

Follow signals carefully; if in doubt about signal do not move the rigging - ask for a repeat. IF IN DOUBT – STOP!

Yarding a Turn:

- Always start the turn off slowly.

- Slow down the turn when coming into the landing area.

- Make sure the crew and the loader are in the clear.

- Keep the landing of the logs as straight as possible.

- Make sure the logs are as secure as possible to minimize the danger of logs rolling onto the chaser.

Other Persons in the Cab:

- No unauthorized person will be in cab while actual yarding is taking place.

- Only authorized persons or those on a job-training program are allowed to run the machine.

Machine Maintenance:

- Good housekeeping is a must; with emphasis on oil spills, accumulations of gear dope and fine material under the drum and winches.

- Keep all guards in place.

- Shut down motor and winches when doing greasing and maintenance work on them.

- Lock-out/de-energize as required.

Spooling of Lines:

- Be sure a proper tool is used.

- No loose or ragged clothing is to be worn when spooling.

Radio Signals:

- Never shut the whistle off until the crew is in the landing.