Safe Procedures

- Conduct a “pre-trip” vehicle check. Use a Vehicle Log to record pre-trip safety inspection.

- Report deficiencies and do not use if equipment is in unsafe condition.

- As the driver you are responsible for your passengers. Ensure that you and all your passengers are wearing seatbelts. No seatbelt – no drive.

- No handheld cell phone use – get a passenger to make and receive your calls, use a Bluetooth device or let it go to message.

- Drive defensively at all times.

- Do not exceed posted speed limits.

- Do not exceed 80kph or posted speed limits on resource roads.

- Drive to the road conditions. Lower speed as required. Consider:

  - Visibility reduced by dust, fog, rain and snow;

  - Narrow roads with over width vehicles;

  - Steep favorable and adverse grades;

  - Slippery and variable road surface conditions due to loose gravel, snow, ice or mud;

  - Other users.

- Use vehicle for intended use only (purpose and weight limitations).

- Drive with vehicle lights on at all times.

- Stay on your side of the road.

- Secure all heavy or sharp objects in the cab of the vehicle.

- Respect that loaded logging trucks have the right of way on single lane roads.

- Do not tailgate

- Pass trucks or equipment only after you receive a clearly visible and/or audible signal from the operator.

- Switch drivers if you are becoming drowsy

- Never chase a runaway vehicle.

Radio Use

- Before starting on road with posted radio channel do a radio check to ensure correct frequency.

- Do not drive by the radio. Expect oncoming traffic on all corners.

- Call your position according to the local radio protocol and signage. Call “up” or “down”

- Notify other radio equipped vehicles of oncoming non-radio equipped traffic.

- Do not use road radio channels for conversations, use only for road traffic protocols.

- Pull over and safely park when talking on the radio phone for an extended period of time. Remember – intense conversations are highly distracting.

Parking

- Park clear of traffic, away from active areas in pullouts or extra wide straight sections of road.

- If radio calling notify traffic that you are clear at x km.

- Park facing the direction of exit with access for service/towing activities.

- Ensure the parking brake is on and the transmission is in 1st gear or park before exiting vehicle.

- On steep grades, use wheel chocks and always turn the wheels towards the nearest ditch.

- Never park on a curve especially on the outside curve of a road.

- When turning around, back into the cut bank of the road and not towards the outside bank.

- Use flares/safety triangles/4 way flashers where required.