Personal Protective Equipment

- Hi-vis hardhat with face shield

- Hi-vis vest

- Leg protection

- Hearing protection

- Caulked boots

- Gloves

SAFE Procedures

- Bucker is to control traffic on landings.

- Ensure other workers are clear of your hazardous area.

- Watch for pivot points and scissor logs while bucking.

- Stand on the upper side of log situated on an incline.

- Be certain you are not standing on the whip side.

- Logs must not be partially bucked.

- When limbing, stand with both legs on the opposite side of the log.

- Avoid stepping on chunks.

- Ensure danger trees are removed by qualified worker from around the landing.

- Keep clear of loader and skidding machines.

- Never turn your back to approaching equipment. Watch for trees that may hang-up and pivot while being skidded onto the landing.

- Do not start limbing or bucking until chokers have been removed or bunches spread.

- Make sure logs are stable before bucking.

- Never walk under a log that is being held up by the loader.

- Choose a maintenance area away from other work activity. Go to this safe zone when bucking is completed.

- If all chokers cannot be unhooked, stand well in the clear when the skidder moves ahead to clear the drag.

Chainsaw Safety

- Saws including the chain brakes are to be maintained in good running order.

- Place saw on a firm base when starting.

- Shut off saw when cleaning it.

- Carry fuel in an approved container suitably marked.

- Be aware of hot muffler when servicing and filling.

- Do not smoke when refueling.

- Fit handles on all files; do not carry files in your pocket.

- Shut off saw when carrying it any distance.

- When walking and carrying the saw, keep the bar to the rear to avoid falling on the chain if you trip.

- To reduce the risk of kickbacks, keep a firm grip on the saw.

- Avoid touching limbs or other objects with the tip of the bar.

- Stop chain when moving from cut to cut.