Personal Protective Equipment

- Hi-vis hardhat

- Hearing protection

- Gloves

- Suitable footwear

- Hi-vis clothing

SAFE Procedures

- Check to ensure machine is in safe operating condition before using.

- Wear your seatbelt when operating machine.

- Ensure good housekeeping to prevent slipping or tripping when entering or leaving machine.  All fire extinguishers and other items must be secured in a safe location.  Do not carry loose articles in the cab.

- Do not enter an active falling area.  Stay a minimum of two tree lengths away.  Do not skid trees past a faller or other active falling areas.

- Travel at a safe speed with or without a turn.

- Exercise caution when working on hillsides.

- Make sure chokers and main-line are in safe working condition.

- When winching, align the machine with the direction of the pull.

- Watch for whip action of logs being skidded.

- When entering the landing, make sure buckers and loaders are in clear view and you are given approval to enter.

- Lower the blade and set parking brake before leaving the machine.

- Follow the lock-out procedures while conducting maintenance work on the machine.

- Wear eye protection when cutting cables.

- Skidding On Slopes

- Do not attempt to travel across a slope that is too steep for maintaining proper balance of the machine.

- Confine your travel to straight up and down slopes when steepness is a problem.  Any slope greater than 35% shall not be traveled without specific steep slope safe work procedures in place.  The procedures must be reviewed before operating on steep slopes.

- When traveling across any slope, avoiding running over chunks and stumps because of the increased possibility of upset.

- Keep turn winched up tight to apron or fairlead or to prevent turn from running into back of machine causing balance and steering problems.

- Release the turn when making a tight corner.

- When skidding on side cuts (trails) on steep ground, maintain safe distance from edge of cut in order to prevent sloughing of outer edge and rollover.

- Do not attempt to bulldoze trees that are hanging over or across skid trails on side cuts as they could snap and spring back into operator’s cab.

- When setting a turn lower blade to ground (try to lower blade behind a stump) and set brakes to prevent runaway.

- Use tire chains for traction on steep ground and when slippery.

- Exercise care when using chains so that chunks are not caught and flung up into the cab.

- When coming down steep slopes, make sure the corner of the blade does not hook a stump or rock, causing the machine to swing sideways and subsequently upset.

- Remember if you encounter any unsafe skidding situations in the course of your shift, inform your supervisor and alternate methods will be initiated.

- If at any time the machine becomes unstable, shut it down and request assistance.

Maintenance/Exiting Machine

- Follow the lock-out procedures while conducting maintenance work on the machine.

- Always enter and leave the machine in a safe manner.  Use the hand holds for stability and beware of slipping hazards that exist, particularly in winter.  (Use the 3 point mount/dismount technique).

| Skidder Lockout -Tagout (If one person working on machine) | Skidder Lockout -Tagout (If more than one person working on machine) | Skidder Tagout For skidder without master switch |
| --- | --- | --- |
| Shut down procedure: | Shut down procedure: | Shut down procedure: |
| - Notify other affected employees. - Apply parking brake. - Lower grapple and blade to ground. - Shut down engine. - Turn off master switch. - Put tag on master switch. - Test to verify zero energy (electrical-hydraulic-gravity). | - Notify other affected employees. - Apply parking brake. - Lower grapple and blade to ground. - Shut down engine. - Turn off master switch. - Place tag on master switch. - Test to verify zero energy (electrical-hydraulic-gravity). | - Notify other affected employees. - Apply parking brake. - Lower grapple and blade to ground. - Shut down engine. - Key out and in pocket. - Put lockout tag initialed by all workers on ignition switch. - Test to verify zero energy (electrical-hydraulic-gravity). |
| Start-up procedure: | Start-up procedure: | Start-up procedure: |
| - Turn on master switch and remove tag from master switch. - Start machine. | - Turn on master switch and remove tag from master switch. - Start machine | - Each employee crosses off their initials on lockout tag when their work is completed. - Start machine when all initials on tag crossed off |